rUBot Driver Arduino Nano ESP32

Contents

Component List	2

Specifications and Considerations	2

Pin Connections	3

Schematic and PCB	3

# Component List

| Component | Name | Quantity | Link |
| --- | --- | --- | --- |
| Connectors Motor | Male connector JST, 6via, 2mm | 4 | https://es.rs-online.com/web/p/conectores-macho-para-pcb/8201447 |
| Component Connectors | Female 20 pin connector, 2.54mm | 5 | https://es.rs-online.com/web/p/conectores-hembra-para-pcb/2053211?gb=s |
| 12V Jack Connector | DC Connector PCB 12V,5A | 1 | https://es.rs-online.com/web/p/conectores-de-alimentacion-dc/8051699 |
| 12 and 5V Connectors | Female 2 pin thermal block, 5.08mm | 2 | https://es.rs-online.com/web/p/bornes-para-pcb/1930564?gb=s |
| USB C Module Connector | Female Connector | 1 | https://es.rs-online.com/web/p/conectores-hembra-para-pcb/6816832?gb=s |
| Motor Driver Module | TB6612FNG driver module | 2 | https://www.amazon.es/unidades-TB6612FNG-controlador-Microcontrolador-Reemplazar/dp/B07NSV8XHK |
| Arduino Nano ESP32 | Arduino Nano ESP32º | 1 | https://es.rs-online.com/web/p/arduino/2686962?gb=s |
| IMU | MPU-9250 IMU | 1 | IMU |
| USB-C Module | USB-C PCB module | 1 | USB C Module |
| SMD USB-C Resistor | SMD 1206 5.1kΩ resistors for the USB-C | 2 | https://es.rs-online.com/web/p/resistencias-de-montaje-en-superficie/6792247?gb=s |
| SMD IMU Resistor | SMD 1206 4.7kΩ resistors for SDA/SCL | 2 | https://es.rs-online.com/web/p/resistencias-de-montaje-en-superficie/7219872P?gb=s |
| SMD IMU Capacitor | Decoupling SMD 1206 100nF Capacitor | 1 | https://es.rs-online.com/web/p/condensadores-ceramicos-multicapa/6698515?gb=s |
| Power Supply Jumper | 2 pin female Jumper | 1 | https://es.rs-online.com/web/p/jumpers-y-derivadores/7457080?gb=s |

Table 1. List of components for the motor driver

# Specifications and Considerations

The board has two DC voltage sources:

- 12 V Power Supply: It can be provided through the barrel jack connector or the thermal block located next to it. It’s the alimentation for the Motor modules to provide the signals of the phases for the motors and it also powers up the Arduino Nano ESP32 through the VIN pin since it can be provided between 5 and 18 V.

- 5 V Power supply: It can be provided through and USB-C module that can be connected on the 2 row 6 pin female connector or the thermal block connected to it’s side. It will be used as the alimentation of the logic part of the Motor drivers and the motor encoders.

Caution: When the Arduino Nano is not connected via USB we need to put the jumper to power the Arduino, do not connect  the jumper and the Arduino via USB at the same time.

The board is two layered and has the following dimensions 70 x 79 mm.

The power consumptions of the components are:

- Arduino Nano ESP32: 274.19 mA (maximum value for simple WIFI code)

- TB6612FNGs: Can provide up to 1.2A continuous current for channel. (4 channels 4.8A)

- IMU: If the 9-axis (no DMP): 1 kHz gyro ODR, 4 kHz accel ODR, 8 Hz mag in repetition rate is used the power consumption will be 3.7 mA.

The components are located so for one side we have all the power supply inputs and the other side the motor outputs and the USB port of the Arduino Nano ESP32.

# Pin Connections

Arduino Nano ESP32 with motor driver modules

| Pin | Motor 1 | Motor 2 | Motor 3 | Motor 4 |
| --- | --- | --- | --- | --- |
| PWM | A3(4) | A7(14) | D2(5) | D7(10) |
| DIR1 | A6(13) | TX1(43) | D4(7) | D5(8) |
| DIR2 | D8(17) | RX0(44) | D3(6) | D6(9) |
| Enc1 | D13(48) | A1(2) | D12(47) | D10(21) |
| Enc2 | A0(1) | A2(3) | D11(38) | D9(18) |

Table 2. List of the Arduino Nano pins for the motor driver module and motor I/O.

Other Connections:

- IMU SDA and SCL: They are connected to the respective A4(11) and A5(12) pins of the Arduino Nano ESP32

- IMU +3.3V alimentation: Is connected to the respective 3.3V output of the Arduino Nano ESP32.

# Schematic and PCB

Fig.1 Schematic of the circuit and components of the rUBot driver Arduino

Fig.2 PCB Design of the rUBot driver Arduino

Fig.3 3D View of the PCB design